/*

HEADER:		CUG267;

TITLE:		8085 Cross-Assembler (Portable);

FILENAME:	A04.DOC;

VERSION:	0.1 as A85, 08/27/1988;

0.2 as A04, Oct 26 2019

0.21 July 2020

SEE-ALSO:	A04.H;

AUTHORS:	William C. Colley, III;

UPDATE:	Dec 2013, changed to compile with LCC under Windows

Nov 2014, changed to support Intel 4004, 4040

Dec 2014, changes to accomdate various assemblers

Oct 2019, some changes done & undone

-  '!' seperates instructions

- relative branch numeric addresses work but flagged

- FIM operand truncated to 8 bits w/o error flagged

July 2020 pseudo-op RPAIRS as evens or 0-7 for doubleregs

-- Herbert R. Johnson

*/

4004 4040  Cross-Assembler (Portable)

converted from 8085 Cross-Assembler by Herb Johnson Oct 2019

Version 0.2

Copyright (c) 1985,1987 William C. Colley, III

The manual such as it is.

Legal Note:    This package may be used for any commercial or

non-commercial purpose.  It may be copied and

distributed freely provided that any fee charged

by the distributor of the copy does not exceed the

sum of:  1) the cost of the media the copy is

written on,  2) any required costs of shipping the

copy, and  3) a nominal handling fee.  Any other

distribution requires the written permission of

the author.  Also, the author's copyright notices

shall not be removed from the program source, the

program object, or the program documentation.

Table of Contents

1.0  How to Use the Cross-Assembler Package ..................  3

2.0  Format of Cross-Assembler Source Lines ..................  4

2.0.1 '!' statement seperator

2.1  Labels .............................................  5

2.2  Numeric Constants ..................................  5

2.3  String Constants ...................................  6

2.4  Expressions ........................................  6

3.0  Machine Opcodes .........................................  7

3.1  Opcodes -- No Arguments ............................  7

3.2  Opcodes -- One Register Argument ...................  7

3.3  Opcodes -- One Address Argument .................     7

3.4  Opcodes -- One Register and One Address Argument      7

3.45 Opcodes -- One Register and one data argument

3.5  Opcodes -- One condition, one address      .........  7

3.6  Comments on seperating operands, identifing registers ?

4.0  Pseudo Opcodes ..........................................  8

4.1  Pseudo-ops -- DB ...................................  9

4.2  Pseudo-ops -- DS ...................................  9

4.3  Pseudo-ops -- DW ...................................  9

4.4  Pseudo-ops -- END .................................. 10

4.5  Pseudo-ops -- EQU .................................. 10

4.6  Pseudo-ops -- IF, ELSE, ENDIF ...................... 10

4.7  Pseudo-ops -- INCL ................................. 11

4.8  Pseudo-ops -- ORG .................................. 12

4.9  Pseudo-ops -- PAGE ................................. 12

4.N  Pseudo-ops -- RPAIRS

4.10 Pseudo-ops -- SET .................................. 12

4.11 Pseudo-ops -- TITLE ................................ 12

5.0  Assembly Errors ......................................... 13

5.1  Error * -- Missing Statement ....................... 13

5.2  Error ( -- Parenthesis Imbalance ................... 13

5.3  Error " -- Missing Quotation Mark .................. 13

5.B  Error B -- Illegal branch outside page

5.4  Error D -- Illegal Digit ........................... 13

5.5  Error E -- Illegal Expression ...................... 13

5.6  Error I -- IF-ENDIF Imbalance ...................... 14

5.7  Error L -- Illegal Label ........................... 14

5.8  Error M -- Multiply Defined Label .................. 14

5.N  Error N -- "Illegal" branch to next page

5.9  Error O -- Illegal Opcode   ........................ 14

5.10 Error P -- Phasing Error ........................... 15

5.11 Error R -- Illegal Register (not checked) .......... 15

5.12 Error S -- Illegal Syntax .......................... 15

5.13 Error T -- Too Many Arguments ...................... 15

5.14 Error U -- Undefined Label ......................... 15

5.15 Error V -- Illegal Value ........................... 15

6.0  Warning Messages ........................................ 16

6.1  Warning -- Illegal Option Ignored .................. 16

6.2  Warning -- -l Option Ignored -- No File Name ....... 16

6.3  Warning -- -o Option Ignored -- No File Name ....... 16

6.4  Warning -- Extra Source File Ignored ............... 16

6.5  Warning -- Extra Listing File Ignored .............. 16

6.6  Warning -- Extra Object File Ignored ............... 16

1

7.0  Fatal Error Messages .................................... 16

7.1  Fatal Error -- No Source File Specified ............ 16

7.2  Fatal Error -- Source File Did Not Open ............ 17

7.3  Fatal Error -- Listing File Did Not Open ........... 17

7.4  Fatal Error -- Object File Did Not Open ............ 17

7.5  Fatal Error -- Error Reading Source File ........... 17

7.6  Fatal Error -- Disk or Directory Full .............. 17

7.7  Fatal Error -- File Stack Overflow ................. 17

7.8  Fatal Error -- If Stack Overflow ................... 17

7.9  Fatal Error -- Too Many Symbols .................... 17

2

1.0  How to Use the Cross-Assembler Package

First, the question, "What does a cross-assembler do?" needs

to be addressed as there is considerable confusion on this point.

A cross-assembler is just like any other assembler except that it

runs on some CPU other than the one for which it assembles code.

For example, this package assembles 4004 & 4040 source code into

object code, but it runs on an 8088, a 68000, or whatever other

CPU you happen to have a C compiler for.  The reason that cross-

assemblers are useful is that you probably already have a CPU

with memory, disk drives, a text editor, an operating system, and

all sorts of hard-to-build or expensive facilities on hand.  A

cross-assembler allows you to use these facilites to develop code

for a 4004 or 4040.

Because this cross-assembler assembles 4004 & 4040 code, it is

possible to use it under CP/M-80 by compiling it

under Aztec C II, Eco-C, etc. and running it under CP/M.

This program requires one input file (your 40XX source code)

and zero to two output files (the listing and the object).  The

input file MUST be specified, or the assembler will bomb on a

fatal error.  The listing and object files are optional.  If no

listing file is specified, no listing is generated, and if no

object file is specified, no object is generated.  If the object

file is specified, the object is written to this file in "Intel

hexadecimal" format.

The command line for the cross-assembler looks like this:

A04 source_file { >list_file } { -o object_file }

where the { } indicates that the specified item is optional.

Some examples are in order:

A04 test04.asm                          source:   test04.asm

listing:  none

object:   none

A04 test04.asm -l test04.prn            source:   test04.asm

listing:  test04.prn

object:   none

A04 test04.asm -o test04.hex            source:   test04.asm

listing:  none

object:   test04.hex

A04 test04.asm -l test04.prn -o test04.hex

source:   test04.asm

listing:  test04.prn

object:   test04.hex

The order in which the source, listing, and object files are

3

specified does not matter.  Note that no default file name exten-

sions are supplied by the assembler as this gives rise to porta-

bility problems.

2.0  Format of Cross-Assembler Source Lines

The source file that the cross-assembler processes into a

listing and an object is an ASCII text file that you can prepare

with whatever editor you have at hand.  The most-significant

(parity) bit of each character is cleared as the character is

read from disk by the cross-assembler, so editors that set this

bit (such as WordStar's document mode) should not bother this

program.  All printing characters, the ASCII TAB character (09H),

and newline character(s) are processed by the assembler.  All

other characters are passed through to the listing file, but are

otherwise ignored.

The source file is divided into lines by newline char-

acter(s).  The internal buffers accommodate lines of up to

255 characters. If you must use longer lines,

change the constant MAXLINE in file A04.H and recompile the

cross-assembler; or you'll crash the assembler.

Each source line is made up of three fields:  the label

field, the opcode field, and the argument field.  The label field

is optional and must begin in column 1. The opcode field is

optional and must not begin in column 1.  If both a label and an

opcode are present, one or more spaces and/or TABs must separate them.

If the opcode requires arguments, they are placed in the argument

field and separated from the opcode field by one or more

spaces and/or TAB characters.

Also, an optional comment can be added to the end of any of the

source-lines described, or as a line without source. Comments begin

with a semicolon; the assembler passes the rest of the line to the

listing file. So a source line looks like this:

{label}{ opcode{ arguments}}{;commentary}

where the { } indicates that the specified item is optional.

column 1

|

GRONK   ISZ   2, POINTER      ; This line has everything.

FIM   0, 12           ; This line has no label.

BEEP                          ; This line has no opcode.

; This line has no label and no opcode.

END                   ; This line has no argument.

2.0.1 '!' statement seperator

I've tried to use a '!' as a statement seperator, to accept

multiple statement lines like:

GRONK	FIM 0,12 ! FIM 1,14 !FIM 2,8

The listing seperates these into individual statement lines. It's

ugly but it works.

4

2.1  Labels

A label is any sequence of alphabetic or numeric characters

starting with an alphabetic. It may end with a colon (:). The

legal alphabetics are:

! # $ % & , . : ?  @ [ \ ] ^ _  ` { | }  ~  A-Z  a-z

The numeric characters are the digits 0-9.  Note that "A" is not

the same as "a" in a label.  This can explain mysterious U

(undefined label) errors occurring when a label appears to be

defined.

A label is permitted on any line except a line where the

opcode is IF, ELSE, or ENDIF.  The label is assigned the value of

the assembly program counter before any of the rest of the line

is processed except when the opcode is EQU, ORG, PAGE, or SET.

Labels can have the same name as opcodes, but they cannot

have the same name as operators or registers.  The reserved

(operator and register) names are:

$         AND       EQ        GE        GT        HIGH

LE        LT        LOW       MOD

NE        NOT       OR        SHL       SHR

XOR

If a label is used in an expression before it is assigned a

value, the label is said to be "forward-referenced."  For

example:

L1   EQU  L2 + 1   ; L2 is forward-referenced here.

L2

L3   EQU  L2 + 1   ; L2 is not forward-referenced here.

2.2  Numeric Constants

Numeric constants are formed according to the Intel

convention.  A numeric constant starts with a numeric character

(0-9), continues with zero or more digits (0-9, A-F), and ends

with an optional base designator.  The base designators are H or $ for

hexadecimal, none or D for decimal, O Q or @ for octal, and B or % for

binary.  The hex digits a-f are converted to upper case by the

assembler.  Note that a numeric constant cannot begin with A-F as

it would be indistinguishable from a label.  Thus, all of the

following evaluate to 255 (decimal):

$FF 0ffH  255 255D 377O 377Q @377  11111111B %11111111

5

2.3  String Constants

A string constant is zero or more characters enclosed in

either single quotes (' ') or double quotes (" ").  Single quotes

only match single quotes, and double quotes only match double

quotes, so if you want to put a single quote in a string, you can

do it like this:  "'".  In all contexts except the DB statement,

the first character or two of the string constant are all that

are used.  The rest is ignored.  Noting that the ASCII codes for

"A" and "B" are 41H and 42H, respectively, will explain the

following examples:

"" and ''           evaluate to 0000H

"A" and 'A'         evaluate to 0041H

"AB"                evaluates to 4142H

Note that the null string "" is legal and evaluates to 0000H.

2.4  Expressions

An expression is made up of labels, numeric constants, and

string constants glued together with arithmetic operators,

logical operators, and parentheses in the usual way that

algebraic expressions are made.  Operators have the following

fairly natural order of precedence:

Highest        anything in parentheses

unary +, unary -

*, /, MOD, SHL, SHR

binary +, binary -

LT, LE, EQ, GE, GT, NE

NOT

AND

OR, XOR

Lowest         HIGH, LOW

A few notes about the various operators are in order:

1)   The remainder operator MOD yields the remainder from

dividing its left operand by its right operand.

2)   The shifting operators SHL and SHR shift their left

operand to the left or right the number of bits

specified by their right operand.

3)   The relational operators LT, LE, EQ, GE, GT, and NE can

also be written as <, <= or =<, =, >= or =>, and <> or

><, respectively.  They evaluate to 0FFFFH if the

statement is true, 0 otherwise.

4)   The logical opeators NOT, AND, OR, and XOR do bitwise

operations on their operand(s).

5)   HIGH and LOW extract the high or low byte, of an

6

expression. Expressions values are limited to 16-bit.

6)   The special symbol $ represents the value of the program

counter before any of the current line has been processed.

Some examples are in order at this point:

2 + 3 * 4                          evaluates to 14

(2 + 3) * 4                        evaluates to 20

NOT 11110000B XOR 00001010B        evaluates to 00000101B

HIGH 1234H SHL 1                   evaluates to 0024H

001Q EQ 0                          evaluates to 0

001Q = 2 SHR 1                     evaluates to 0FFFFH

All arithmetic is unsigned with overflow from the 16-bit

word ignored.  So "32768 * 2" evaluates to 0.

3.0  Machine Opcodes

The opcodes of the 40XX processor are divided into groups

below by the type of arguments required in the argument field of

the source line.  If an opcode requires multiple arguments, these

must be placed in the argument field in order and separated by

commas. A04 will not accept spaces between arguments! The arguments

for register pairs must resolve to 0 to 7, or for registers 0 to 15;

A04 will otherwise flag these as errors. Likewise A04 will test for

local (8-bit) jumps of the argument is outside the current program page.

The following opcodes list may be incomplete; refer to processor

documentation for details.

3.1  Opcodes -- No Arguments

These 4040 opcodes allow no arguments in their  argument fields:

HLT BBS	LCR	OR4	OR5	AN6	AN7	DB0 	DB1	SB0 	SB1

EIN DIN 	RPM

Likewise for these 4004 opcodes:

WRM WMP WRR WPM WR0 WR1 WR2 WR3 SBM RDM RDR ADM RD0 RD1 RD2 RD3

CLB CLC IAC CMC CMA RAL RAR TCC DAC TCS STC DAA KBP DCL

3.2  Opcodes -- One Register Argument

The opcodes in this group require one argument that is a

register value, or a register pair value.  You may wish to EQU

specific mnemonics as register-values. The opcodes are:

Allowable Registers     Opcodes

pairs 0-7			SRC FIN JIN FIM

registers 0-15		INC ADD SUB LD XCH

Note: Intel's 4004 assembler requires even-values for register-pairs

0 2 4 6 8 10 12 14. A04 currently requires 0 for R0&1, 1 for R2 and R3,

etc. up to 7 for R14 and R15; larger values are errors. You can use

EQUated mnemonics for register pairs, or workarounds like R15/2. - Herb

7

3.3  Opcodes -- One Address Argument

The following opcodes require one address argument as follows:

One 12-bit address: JUN JMS

One 8-bit address: JT JNT JC JCN JZ JNZ (as JCN instructions).

3.4  Opcodes -- One Register and One Address Argument

ISZ - one register 0-15, one 8-bit address

3.45 Opcodes -- One Register and one data argument

FIM - one register pair, one 8-bit data. Data value truncated

to 8-bits without an error flagged. Register pair value 0 thru

7, see section 3.2 above.

3.5  Opcodes -- one condition and one Address

JCN requires a condition value from 0 to 15, and an 8-bit address.

A04 has an "equates.asm" file for suggested condition mnemonics.

7.5

3.6  Comments on seperating operands, identifing registers

The A04 assembler is a typical assembler of the 1980's, with a syntax

roughly as follows:

lable<:>     opcode  <operand>,<operand>      <;comment>

Where a lable may or may not have a colon, the opcode is separated from its

operands by spaces or tab, and operands are separated by commas. Lables and

values expressed as text strings, MUST start with a non-numeric character;

constant values MUST start with a numeric value or radix character.

Some 4004 assemblers may use other syntaxes. Intel's early 4004 assembly

language documents use no comma between two operands. Also:

some operands in earlier assemblers may begin with a numeric digit. One

Intel assembler defined register pairs as 0P, 1P, etc. Other assemblers,

or hand-assembly, of 4004/4040 code may designate registers and register

pairs by other strings.

As a matter of convenience, or lack of understanding of code, I've made

the following choices in this A04 assembler as converted from a "conventional"

8085 assembler:

There are no predefined "names" for registers. The user should add an "include"

file (I provide equates.asm) which defines your choice of text values to represent

single registers 0-15 and register pairs 0-7. They must start with an alphabetic.

Two-operand instructions must use a commas to separate the operands.

There's likely other changes needed to a 4004 or 4040 source to avoid syntax

conflicts. I found these changes can be down with the simplest of text-editor

"find and replace" commands. Tedious, but doable. More elaborate text editors have

"grep" commands to expedite those changes. I don't believe most sources will be

overly compromized by making these textual changes. The C sources are available

if a user wants to change the A04 syntax; I've be glad to hear about any such

changes, or consider making them myself if it's practical, feasible, etc.

Herb Johnson

8

4.0  Pseudo Opcodes

Unlike 40XX opcodes, pseudo opcodes (pseudo-ops) do not

represent machine instructions.  They are, rather, directives to

the assembler.  These directives require various numbers and

types of arguments.  They will be listed individually below.

4.1  Pseudo-ops -- DB

The DB pseudo-op allows arbitrary bytes to be spliced into

the object code.  Its argument is a chain of zero or more

expressions that evaluate to -128 through 255 separated by

commas.  If a comma occurs with no preceding expression, a 00H

byte is spliced into the object code.  The sequence of bytes

0FEH, 0FFH, 00H, 01H, 02H are produced by the following statement.

DB        -2, -1, , 1, 2

Consider that some register use may make it convenient to use

hexidecimal notation to create BCD *decimal* nibble digits.

To conform to Intel usage, string constants are handled

differently in DB statements than in other places.  If a string

constant is written with no other arithmetic done on it, the

string constant is not truncated to two bytes.  Instead, the

entire string is be spliced into the object code.  For example:

DB        "Kaboom!!", 0DH, 0AH     ;This is 10

;bytes of code.

4.2  Pseudo-ops -- DS

The DS pseudo-op is used to reserve a block of storage for

program variables, or whatever.  This storage is not initialized

in any way, so its value at run time will usually be random.  The

argument expression (which may contain no forward references) is

added to the assembly program counter.  The following statement

would reserve 10 bytes of storage called "STORAGE":

STORAGE   DS        10

4.3  Pseudo-ops -- DW

The DW pseudo-op allows 16-bit words to be spliced into the

object code.  Its argument is a chain of zero or more expressions

9

separated by commas.  If a comma occurs with no preceding

expression, a word of 0000H is spliced into the code.  The word

is placed into memory low byte in low address, high byte in high

address as per standard Intel order.  The sequence of bytes

0FFH, 0FEH, 00H, 00H, 02H, 01H could be spliced into the code

with the following statement:

DW        0FEFFH, , 0102H

4.4  Pseudo-ops -- END

The END pseudo-op tells the assembler that the source

program is over.  Any further lines of the source file are

ignored and not passed on to the listing.  If an argument is

added to the END statement, the value of the argument will be

placed in the execution address slot in the Intel hex object

file.  The execution address defaults to the program counter

value at the point where the END was encountered.  Thus, to

specify that the program starts at label START, the END statement

would be:

END       START

If end-of-file is encountered on the source file before an

END statement is reached, the assembler will add an END statement

to the listing and flag it with a * (missing statement) error.

4.5  Pseudo-ops -- EQU

The EQU pseudo-op is used to assign a specific value to a

label, thus the label on this line is REQUIRED.  Once the value

is assigned, it cannot be reassigned by writing the label in

column 1, by another EQU statement, or by a SET statement.  Thus,

for example, the following statement assigns the value 2 to the

label TWO:

TWO       EQU       1 + 1

The expression in the argument field must contain no forward

references.

4.6  Pseudo-ops -- IF, ELSE, ENDIF

These three pseudo-ops allow the assembler to choose whether

or not to assemble certain blocks of code based on the result of

an expression.  Code that is not assembled is passed through to

the listing but otherwise ignored by the assembler.  The IF

pseudo-op signals the beginning of a conditionally assembled

block.  It requires one argument that may contain no forward

references.  If the value of the argument is non-zero, the block

is assembled.  Otherwise, the block is ignored.  The ENDIF

pseudo-op signals the end of the conditionally assembled block.

10

For example:

IF   EXPRESSION     ;This whole thing generates

DB   01H, 02H, 03H  ;  no code whatsoever if

ENDIF               ;  EXPRESSION is zero.

The ELSE pseudo-op allows the assembly of either one of two

blocks, but not both.  The following two sequences are

equivalent:

IF   EXPRESSION

... some stuff ...

ELSE

... some more stuff ...

ENDIF

TEMP_LAB  SET  EXPRESSION

IF   TEMP_LAB NE 0

... some stuff ...

ENDIF

IF   TEMP_LAB EQ 0

... some more stuff ...

ENDIF

The pseudo-ops in this group do NOT permit labels to exist

on the same line as the status of the label (ignored or not)

would be ambiguous.

All IF statements (even those in ignored conditionally

assembled blocks) must have corresponding ENDIF statements and

all ELSE and ENDIF statements must have a corresponding IF

statement.

IF blocks can be nested up to 16 levels deep before the

assembler dies of a fatal error.  This should be adequate for any

conceivable job, but if you need more, change the constant

IFDEPTH in file A04.H and recompile the assembler.

4.7  Pseudo-ops -- INCL

The INCL pseudo-op is used to splice the contents of another

file into the current file at assembly time.  The name of the

file to be INCLuded is specified as a normal string constant, so

the following line would splice the contents of file "const.def"

into the source code stream:

INCL      "const.def"

INCLuded files may, in turn, INCLude other files until four

files are open simultaneously.  This limit should be enough for

any conceivable job, but if you need more, change the constant

FILES in file A04.H and recompile the assembler.

11

4.8  Pseudo-ops -- ORG

The ORG pseudo-op is used to set the assembly program

counter to a particular value.  The expression that defines this

value may contain no forward references.  The default initial

value of the assembly program counter is 0000H.  The following

statement would change the assembly program counter to 0F000H:

ORG       0F000H

If a label is present on the same line as an ORG statement,

it is assigned the new value of the assembly program counter.

4.9  Pseudo-ops -- PAGE

The PAGE pseudo-op always causes an immediate page ejection

in the listing by inserting a form feed ('\f') character before

the next line.  If an argument is specified, the argument

expression specifies the number of lines per page in the listing.

Legal values for the expression are any number except 1 and 2.  A

value of 0 turns the listing pagination off.  For 60-line pages:

PAGE      60

4.n  Pseudo-ops -- RPAIRS

The RPAIRS pseduo-op has only two argument values: "EVENS" and "ZERO7".

"EVENS" tells the assembler that double-register symbols and values are

0 2 4 6 8 10 12 14; "ZERO7" are 0 thru 7 inclusive. Assemblers for the

4004 and 4040 used either means to designate register-pairs for the

instructions SRC FIM FIN JIN. To designate even-values:

RPAIRS "EVENS"

4.10 Pseudo-ops -- SET

The SET pseudo-op functions like the EQU pseudo-op except

that the SET statement can reassign the value of a label that has

already been assigned by another SET statement.  Like the EQU

statement, the argument expression may contain no forward

references.  A label defined by a SET statement cannot be

redefined by writing it in column 1 or with an EQU statement.

The following series of statements would set the value of label

"COUNT" to 1, 2, then 3:

COUNT     SET       1

COUNT     SET       2

COUNT     SET       3

4.11 Pseudo-ops -- TITLE

The TITL pseudo-op sets the running title for the listing.

The argument field is required and must be a string constant,

though the null string ("") is legal.  This title is printed

after every page ejection in the listing, therefore, if page

ejections have not been forced by the PAGE pseudo-op, the title

will never be printed.  The following statement would print the

title "Random Bug Generator -- Ver 3.14159" at the top of every

page of the listing:

12

TITL      "Random Bug Generator -- Ver 3.14159"

5.0  Assembly Errors

When a source line contains an illegal construct, the line

is flagged in the listing with a single-letter code describing

the error.  The meaning of each code is listed below.  In

addition, a count of the number of lines with errors is kept and

printed on the C "stderr" device (by default, the console) after

the END statement is processed.  If more than one error occurs in

a given line, only the first is reported.  For example, the

illegal label "=$#*'(" would generate the following listing line:

L  0000   FF 00 00      =$#*'(     LDA       R0

5.1  Error * -- Illegal or Missing Statement

This error occurs when either:

1)   the assembler reaches the end of the source file

without seeing an END statement, or

2)   an END statement is encountered in an INCLude file.

If you are "sure" that the END statement is present when the

assembler thinks that it is missing, it probably is in the

ignored section of an IF block.  If the END statement is missing,

supply it.  If the END statement is in an INCLude file, delete

it.

5.2  Error ( -- Parenthesis Imbalance

For every left parenthesis, there must be a right.

5.3  Error " -- Missing Quotation Mark

Strings have to begin and end with either " or '.  Remember

that " only matches " while ' only matches '.

5.B  Error B -- Illegal branch outside of page

Occurs if branch expression is to another page of code, outside of the

page currently containing the opcode. A page is the memory addressed by

the high-byte of the address expression: 0000H, 0100H, 0200H, and so on.

Note: assembler produces low-order byte of address expression. Some

disassemblies of binary code, produce only the low-order address byte in

disassembled source - the same byte found in the binary. In that context,

this error is a "warning" - but watch for actual erronous branching.

5.4  Error D -- Illegal Digit

This error occurs if a digit greater than or equal to the

base of a numeric constant is found.  For example, a 2 in a

binary number would cause a D error.  Especially, watch for 8 or

9 in an octal number.

5.5  Error E -- Illegal Expression

13

This error occurs because of:

1)   a missing expression where one is required

2)   a unary operator used as a binary operator or vice-

versa

3)   a missing binary operator

4)   a SHL or SHR count that is not 0 thru 15

5.6  Error I -- IF-ENDIF Imbalance

For every IF there must be a corresponding ENDIF.  If this

error occurs on an ELSE or ENDIF statement, the corresponding IF

is missing.  If this error occurs on an END statement, one or

more ENDIF statements are missing.

5.7  Error L -- Illegal Label

This error occurs because of:

1)   a non-alphabetic in column 1

2)   a reserved word used as a label

3)   a missing label on an EQU or SET statement

4)   a label on an IF, ELSE, or ENDIF statement

5.8  Error M -- Multiply Defined Label

This error occurs because of:

1)   a label defined in column 1 or with the EQU statement

being redefined

2)   a label defined by a SET statement being redefined

either in column 1 or with the EQU statement

3)   the value of the label changing between assembly passes

5.N  Error N -- Illegal jump or reference to next page

JIN or FIN that is assembled at the end of the page,

at XXFFH, will if executed reference the NEXT page of code. ISZ or JCN

instructions, if assembled at the end of the page at XXFEH or XXFFH,

will if executed branch within the NEXT page of memory. These will

either cause odd effects or odd jumps or be confusing. Consult the

4004 and 4040 manuals for details.

5.9  Error O -- Illegal Opcode

The opcode field of a source line may contain only a valid

machine opcode, a valid pseudo-op, or nothing at all.  Anything

else causes this error.

14

5.10 Error P -- Phasing Error

This error occurs because of:

1)   a forward reference in a DS, EQU, ORG, or SET statement

2)   a label disappearing between assembly passes

5.11 Error R -- Illegal Register

This error occurs when a register name is used with an

instruction that cannot reference that register. (However, this

version of the 4040/4004 assembler doesn't check "names", just

numeric values for registers or register pairs. If a numeric value

is out of range for a register designation, a 'V' error is generated.)

5.12 Error S -- Illegal Syntax

This error means that an argument field is scrambled.  Sort

the mess out and reassemble.

5.13 Error T -- Too Many Arguments

This error occurs if there are more items (expressions,

register designators, etc.) in the argument field than the opcode

or pseudo-op requires.  The assembler ignores the extra items but

issues this error in case something is really mangled.

5.14 Error U -- Undefined Label

This error occurs if a label is referenced in an expression

but not defined anywhere in the source program.  If you are

"sure" you have defined the label, note that upper and lower case

letters in labels are different.  Defining "LABEL" does not

define "Label."

5.15 Error V -- Illegal Value

This error occurs because:

1)   an immediate value is not -128 thru 255, or

2)   a DB argument is not -128 thru 255, or

3)   a register number is not 0 thru 15, or

4)   a double register value is either not 0-7 or not even, or

5)   an INCL argument refers to a file that does not exist.

15

6.0  Warning Messages

Some errors that occur during the parsing of the cross-

assembler command line are non-fatal.  The cross-assembler flags

these with a message on the C "stdout" device (by default, the

console) beginning with the word "Warning."  The messages are

listed below:

6.1  Warning -- Illegal Option Ignored

The only options that the cross-assembler knows are -l and

-o.  Any other command line argument beginning with - will draw

this error.

6.2  Warning -- -l Option Ignored -- No File Name

6.3  Warning -- -o Option Ignored -- No File Name

The -l and -o options require a file name to tell the

assembler where to put the listing file or object file.  If this

file name is missing, the option is ignored.

6.4  Warning -- Extra Source File Ignored

The cross-assembler will only assemble one file at a time,

so source file names after the first are ignored.  To assemble a

second file, invoke the assembler again.  Note that under CP/M-

80, the old trick of reexecuting a core image will NOT work as

the initialized data areas are not reinitialized prior to the

second run.

6.5  Warning -- Extra Listing File Ignored

6.6  Warning -- Extra Object File Ignored

The cross-assembler will only generate one listing and

object file per assembly run, so -l and -o options after the

first are ignored.

7.0  Fatal Error Messages

Several errors that occur during the parsing of the cross-

assembler command line or during the assembly run are fatal.  The

cross-assembler flags these with a message on the C "stdout"

device (by default, the console) beginning with the words "Fatal

Error."  The messages are explained below:

7.1  Fatal Error -- No Source File Specified

This one is self-explanatory.  The assembler does not know

what to assemble.

16

7.2  Fatal Error -- Source File Did Not Open

The assembler could not open the source file.  The most

likely cause is that the source file as specified on the command

line does not exist.  On larger systems, there could also be

priviledge violations.  Rarely, a read error in the disk

directory could cause this error.

7.3  Fatal Error -- Listing File Did Not Open

7.4  Fatal Error -- Object File Did Not Open

This error indicates either a defective listing or object

file name or a full disk directory.  Correct the file name or

make more room on the disk.

7.5  Fatal Error -- Error Reading Source File

This error generally indicates a read error in the disk data

space.  Use your backup copy of the source file (You do have one,

don't you?) to recreate the mangled file and reassemble.

7.6  Fatal Error -- Disk or Directory Full

This one is self-explanatory.  Some more space must be found

either by deleting files or by using a disk with more room on it.

7.7  Fatal Error -- File Stack Overflow

This error occurs if you exceed the INCLude file limit of

four files open simultaneously.  This limit can be increased by

increasing the constant FILES in file A04.H and recompiling the

cross-assembler.

7.8  Fatal Error -- If Stack Overflow

This error occurs if you exceed the nesting limit of 16 IF

blocks.  This limit can be increased by increasing the constant

IFDEPTH in file A04.H and recompiling the cross-assembler.

7.9  Fatal Error -- Too Many Symbols

Congratulations!  You have run out of memory.  The space for

the cross-assembler's symbol table is allocated at run-time using

the C library function calloc(), so the cross-assembler will use

all available memory.  The only solutions to this problem are to

lessen the number of labels in the source program or to add more

memory.

17

##########################################################################################################